|  |  | MATERIAL SAFETY DATA SHEET SECTION 1 - PRODUCT IDENTIFICATION AND USE. PRODUCT IDENTIFIER: - Aerosol Foam; PRODUCT IDENTIFICATION NUMBER(PIN): 33800, 33900. PRODUCT USE: Metal Polishing Aid. MANUFACTURERES NAME: Tri-Peek International Inc.; SUPPLIERS NAME:. STREET ADDRESS: 390 Bradwick Dr., 1st Floor; STREET ADDRESS:. CITY: Concord; PROVENCE: Ontario; CITY:; PROVENCE:. POSTAL CODE: L4K 2H7; EMERGENCY TELEPHONE NO: (905) 660-9613; POSTAL CODE:; EMERGENCY TELEPHONE NO:. SECTION 2 - HAZARDOUS INGREDIENTS. HAZARDOUS INGREDIENTS; %; CAS NUMBER; LD50 OF INGREDIENT (SPECIFY SPECIES AND ROUTE); LC50 OF INGREDIENT (SPECIFY SPECIES). Petroleum Naptha; 5-9; 8008-20-6; > 5000mg/Kg Oral, Rat. Petroleum Naptha; 1-4; 64742-48-9; Not Established. Aluminum Oxide; 7-12; 1344-28-1; Not Available. Isobutane - Propane Blend; 5-7; 68476-86-6; 6960mg/Kg Oral, Rat.. SECTION 3 - PHYSICAL DATA. PHYSICAL STATE: Liquid; ODOUR AND APPEARANCE Pleasant Odour / Blue; ODOURTHRESHOLD (ppm) :. VAPOUR PRESSURE (mm Hg): 27-30; VAPOUR DENSITY (AIR=1): 2.07-4.8; EVAPORATION RATE: No Data; BOILING POINT(C) : 93; FREEZING POINT(C): < 0. pH: 8-9; SPECIFIC GRAVITY: 1.072; COEFF. WATER/OIL DIST.: Oil Distribution: > 1.0.. SECTION 4 - FIRE AND EXLOSION DATA. FLAMABILITY: YES NO , IF YES UNDER WHICH CONDITIONS>. MEANS OF EXTINCTION: Use Foam, Dry Chemical, CO2, Water Fog to extinguish fire. Keep fire exposed aerosols with cool water.. FLASHPOINT (C) AND METHOD: 11.6 TO 40.6 (Estimate); UPPER FLAMABLE LIMIT (% BY VOLUME): 12.0; LOWER FLAMMABLE LIMIT (% BY VOLUME): 1.0. AUTOIGNITION TEMPERATURE(C): 250 to 456 (Estimate); HAZARDOUS COMBUSTION PRODUCTS: Oxides of carbon, and other hydrocarbon fumes.. EXPLOSION DATA:; SENSITIVITY TO IMPACT: Do not impact aerosol cans; SENSITIVITY TO STATIC DISCHARGE: No Data... SECTION 5 - REACTIVITY DATA. CHEMICAL STABILITY YES NO. INCOMPATABILITY WITH OTHER SUBSTANCES? YES NO; IF SO, UNDER WHICH ONES?; Avoid excessive ammounts of heat, and all possible ignition sources. Avoid strong oxidizers & acids.. IF NO, UNDER WHICH CONDITIONS?.. REACTIVITY, AND UNDER WHAT CONDITIONS:. HAZARDOUS DECOMPOSTIONS PRODUCTS Carbon monoxide, carbon dioxide, and other irritating gases... SECTION 6 - TOXICOLOGICAL PROPERTIES. ROUTE OF ENTRY SKIN CONTACT , SKIN ABSORPTION , EYE CONTACT , INHILATION , INGESTION. EFFECTS OF ACUTE EXPOSER TO PRODUCT: Skin: Mild irritation may result with prolonged contact. Ingestion: May cause irritation & burning of mouth & throat. EFFECTS OF CHRONIC EXPOSURE TO PRODUCT:. EXPOSURE LIMITS: Not Established; IRRITANCY OF PRODUCT: May be irritant to some skin; SENSITIZATION TO PRODUCT: May be sensitive to some skin; CARCINOGENICITY:. TERATOGENICITY:; REPRODUCTIVE TOXICITY: None Known; MUTAGENICITY:; CYNERGISTIC PRODUCTS: None Known.. SECTION 7 - PREVENTATIVE MEASURES. PERSONAL PROTECTIVE EQUIPMENT: Use proper ventilation if used indoors or in confined areas continuously.. GLOVES (SPECIFY): Not normally required; REPIRATOR (SPECIFY): Not normally required; EYE (SPECIFY): Goggles. FOOTWEAR (SPECIFY): Boots; CLOTHING (SPECIFY): Apron; OTHER (SPECIFY). ENGINEERING CONTROLS (SPECIFY, EG. VENTILATION, ENCLOSED PROCESS.) Standard Ventilation required.. LEAK AND SPILL PROCEDURE: Eliminate source of ignition. Stop or reduce discarge if safe to do so. Absorb spills with clay or diatomaceuos earth.. WASTE DISPOSAL: In accordance with provincial & Federal regulations. Do not dispose in sewers. HANDLING PROCEDURES AND EQUIPMENT: Do not use in poorly ventilated areas.. STORAGE REQUIREMENTS: Store in cool, well ventilated areas. Do not expose to temps > 49 degrees celsius. SPECIAL SHIPPING INFORMATION: Aerosols, keep from freezing. WHMIS Class A, Class D, Div 1, Sub Div 8... SECTION 8 - FIRST AID MEASURES. SPECIFIC MEASURES:. SKIN:; Wipe off immediately, then wash with soap & water. EYES:; Flush with running water for a minimum of 20 minutes. If irritation presists , consult physician. INHILATION:; If overcome by fumes, move victim to frsh air. Obtain medical attention.. INGESTION:; Do not induce vomiting. Rinse out mouth and give 1/2 to 1 cup of water to dilute material. Consuly physician... SECTION 9 - PREPERATION DATE OF MSDS. PREPARED BY (GROUP, DEPARTMENT, ETC.): Tri-Peek Safety Committee; PHONE NUMBER: Tri-Peek Safety Committee; DATE: January 1, 2002.. MATERIAL SAFETY DATA SHEET SECTION 1 - PRODUCT IDENTIFICATION AND USE. PRODUCT IDENTIFIER: - Cream / Paste; PRODUCT IDENTIFICATION NUMBER(PIN): 33100, 33200, 33300,33400, 33500. PRODUCT USE: Metal Polishing Aid. MANUFACTURERES NAME: Tri-Peek International Inc.; SUPPLIERS NAME:. STREET ADDRESS: 390 Bradwick Dr., 1st Floor; STREET ADDRESS:. CITY: Concord; PROVENCE: Ontario; CITY:; PROVENCE:. POSTAL CODE: L4K 2H7; EMERGENCY TELEPHONE NO: (905) 660-9613; POSTAL CODE:; EMERGENCY TELEPHONE NO:. SECTION 2 - HAZARDOUS INGREDIENTS. HAZARDOUS INGREDIENTS; %; CAS NUMBER; LD50 OF INGREDIENT (SPECIFY SPECIES AND ROUTE); LC50 OF INGREDIENT (SPECIFY SPECIES). Petroleum Naptha; 5-9; 8008-20-6; > 5000mg/Kg Oral, Rat. Petroleum Naptha; 1-4; 64742-48-9; Not Established. Aluminum Oxide; 7-12; 1344-28-1; Not Available.. SECTION 3 - PHYSICAL DATA. PHYSICAL STATE: Paste/Cream; ODOUR AND APPEARANCE Pleasant Odour / Blue; ODOURTHRESHOLD (ppm) : No Data. VAPOUR PRESSURE (mm Hg): 7-17.5; VAPOUR DENSITY (AIR=1): 4.8; EVAPORATION RATE: .10; BOILING POINT(C) : 100; FREEZING POINT(C): < 0. pH: N/A; SPECIFIC GRAVITY: >1; COEFF. WATER/OIL DIST.: Oil Distribution: > 1.0.. SECTION 4 - FIRE AND EXLOSION DATA. FLAMABILITY: YES NO , IF YES UNDER WHICH CONDITIONS>. MEANS OF EXTINCTION: Not applicable, Water may be used to cool closed containers to prevent pressure buildup if exporsed to extreme temperature. FLASHPOINT (C) AND METHOD: 40.6 (Estimate); UPPER FLAMABLE LIMIT (% BY VOLUME): 12.0; LOWER FLAMMABLE LIMIT (% BY VOLUME): 1.0. AUTOIGNITION TEMPERATURE(C): 250 to 456 (Estimate); HAZARDOUS COMBUSTION PRODUCTS: Hydrocarbon fumes & smoke. EXPLOSION DATA:; SENSITIVITY TO IMPACT: Container may explode; SENSITIVITY TO STATIC DISCHARGE: No Data... SECTION 5 - REACTIVITY DATA. CHEMICAL STABILITY YES NO. INCOMPATABILITY WITH OTHER SUBSTANCES? YES NO; IF SO, UNDER WHICH ONES?; Strong oxidizers & acids. IF NO, UNDER WHICH CONDITIONS?.. REACTIVITY, AND UNDER WHAT CONDITIONS:. HAZARDOUS DECOMPOSTIONS PRODUCTS Hydrocarbon fumes & smoke. CO where combustion is incomplete.. SECTION 6 - TOXICOLOGICAL PROPERTIES. ROUTE OF ENTRY SKIN CONTACT , SKIN ABSORPTION , EYE CONTACT , INHILATION , INGESTION. EFFECTS OF ACUTE EXPOSER TO PRODUCT: Skin: Mild irritation may result with prolonged contact. Ingestion: May cause irritation & burning of mouth & throat. EFFECTS OF CHRONIC EXPOSURE TO PRODUCT:. EXPOSURE LIMITS: Not Established; IRRITANCY OF PRODUCT: May be irritant to some skin; SENSITIZATION TO PRODUCT: May be sensitive to some skin; CARCINOGENICITY:. TERATOGENICITY:; REPRODUCTIVE TOXICITY: None Known; MUTAGENICITY:; CYNERGISTIC PRODUCTS: None Known.. SECTION 7 - PREVENTATIVE MEASURES. PERSONAL PROTECTIVE EQUIPMENT: Use proper ventilation if used indoors or in confined areas continuously.. GLOVES (SPECIFY): Not normally required; REPIRATOR (SPECIFY): Not normally required; EYE (SPECIFY): Goggles. FOOTWEAR (SPECIFY): Boots; CLOTHING (SPECIFY): Apron; OTHER (SPECIFY). ENGINEERING CONTROLS (SPECIFY, EG. VENTILATION, ENCLOSED PROCESS.) Standard Ventilation required.. LEAK AND SPILL PROCEDURE: Absorb spills with clay or diatomaceous earth.. WASTE DISPOSAL: In accordance with provincial & Federal regulations. Do not dispose in sewers. HANDLING PROCEDURES AND EQUIPMENT: Do not use in poorly ventilated areas.. STORAGE REQUIREMENTS: Store in cool, well ventilated areas. Do not expose to temps > 49 degrees celsius or < 0 degrees celsius. SPECIAL SHIPPING INFORMATION: Keep from freezing.. SECTION 8 - FIRST AID MEASURES. SPECIFIC MEASURES:. SKIN:; Wipe off immediately, then wash with soap & water. EYES:; Flush with running water for a minimum of 20 minutes. If irritation presists , consult physician. INHILATION:; If overcome by fumes, move victim to frsh air. Obtain medical attention.. INGESTION:; Do not induce vomiting. Rinse out mouth and give 1/2 to 1 cup of water to dilute material. Consuly physician... SECTION 9 - PREPERATION DATE OF MSDS. PREPARED BY (GROUP, DEPARTMENT, ETC.): Tri-Peek Safety Committee; PHONE NUMBER: Tri-Peek Safety Committee; DATE: January 19, 2009.. |
| --- | --- | --- |